### Course Code: CSE-360

### Course Title: Microprocessor and Microcontrollers

### Project Name: Calculator For Simple Mathematical Calculation.

### Submitted to:

### Md. Ismail

### Lecturer, UITS,

### B.Sc. in CSE, RUET.

### Submitted by:

### Name : Adibul Asam

### ID : 0432310005101025

### Batch : 53rd

### Section : 6(A1)

### Introduction:

### The purpose of this project is to design and implement a simple arithmetic calculator using 8086 Assembly Language. The calculator runs in a DOS environment and supports four basic arithmetic operations: Addition, Subtraction, Multiplication, and Division.

### The project demonstrates key concepts of assembly language such as interrupt handling, user input/output operations, procedure calls, integer arithmetic instructions, register manipulation, and program control flow.

### This project also helps learners understand how low-level programming interacts directly with the CPU through instructions like add, sub, imul, and idiv.

### Method & Description:

### The calculator program follows a menu-driven approach. The user is first shown a list of operations and is prompted to select one. Each operation is implemented with separate procedures for better modularity.

### Main Components of the Program

### 1. Menu Display

### The menu is printed using DOS interrupt INT 21h, function 09h.
The user selects an option using function 01h.

### 2. Input Handling

### Two numbers are taken as input using the custom procedure read_number.
It handles:

### Multi-digit numbers

### Negative numbers

### Input validation

### Characters are converted to integers by subtracting '0' and constructing the number step by step.

### 3. Arithmetic Operations

### Each arithmetic operation is performed using 8086 instructions:

### Addition: add ax, num2

### Subtraction: sub ax, num2

### Multiplication: imul num2

### Division:

### cwd (sign extension)

### idiv num2

### Division also handles division by zero, displaying an error message.

### Sample Output:

Example 1: Addition

Example 2: Division

Example 3: Division By Zero

### Conclusion

This project successfully demonstrates how a basic calculator can be built using 8086 Assembly Language. By implementing user input/output, arithmetic instructions, procedure handling, and error checking, this project provides a strong foundation in low-level programming concepts.

The program is efficient, modular, and interactive—showing how assembly language can control the CPU directly to perform essential computational tasks.